NETWORK SWITCH SCHEDULING ALGORITHMS

Lab Assignment Report

Evaluating FIFO, VOQ-Optimal, and iSLIP Scheduling on a 3×3 Crossbar Switch

| KEY RESULTS SUMMARY |  |
| --- | --- |
| Algorithm | Total Service Time |
| FIFO (Head-of-Line Blocking) | 11 time slots |
| VOQ — Optimal | 7 time slots |
| VOQ — iSLIP (3 iterations) | 8 time slots |

# 1. Problem Statement

Modern network switches must forward packets from multiple input ports to multiple output ports simultaneously. In a 3×3 crossbar switch with inputs I₀, I₁, I₂ and outputs O₀, O₁, O₂, the central challenge is deciding, at each time slot, which input sends to which output — a problem called switch scheduling.

This assignment evaluates three scheduling approaches:

- Standard FIFO queuing — simple but suffers from Head-of-Line (HoL) blocking

- VOQ Optimal — eliminates HoL blocking via exhaustive search for the best matching

- iSLIP — a practical, hardware-implementable algorithm using round-robin arbitration

The goal is to simulate all three on a common 18-packet input trace, measure the Total Service Time (the time slot when the switch becomes completely empty), and visualise how each algorithm drains its queues over time.

# 2. System Model & Assumptions

- Switch Fabric: 3×3 crossbar (inputs I₀, I₁, I₂ → outputs O₀, O₁, O₂).

- Time Slots: Synchronous operation; each port transmits/receives exactly one packet per slot.

- Propagation: Processing and switching take exactly 1 time slot.

- Packets arrive at the beginning of the designated time slot.

- Tie-Breaking (FIFO): When multiple inputs contend for the same output, the lowest-numbered input port wins (I₀ beats I₁ beats I₂).

## 2.1 Input Trace

The simulation uses the following exact 18-packet arrival sequence:

| Packet ID | Arrival Time (t) | Source Input | Destination Output |
| --- | --- | --- | --- |
| p1 | 0 | I₀ | O₀ |
| p2 | 0 | I₀ | O₁ |
| p3 | 0 | I₁ | O₀ |
| p4 | 0 | I₁ | O₂ |
| p5 | 0 | I₂ | O₀ |
| p6 | 1 | I₀ | O₂ |
| p7 | 1 | I₂ | O₁ |
| p8 | 2 | I₁ | O₁ |
| p9 | 2 | I₂ | O₂ |
| p10 | 3 | I₀ | O₁ |
| p11 | 3 | I₁ | O₀ |
| p12 | 3 | I₂ | O₁ |
| p13 | 4 | I₀ | O₀ |
| p14 | 4 | I₁ | O₂ |
| p15 | 4 | I₂ | O₂ |
| p16 | 5 | I₀ | O₂ |
| p17 | 5 | I₁ | O₁ |
| p18 | 5 | I₂ | O₀ |

# 3. Part 1 — Standard FIFO with HoL Blocking

## 3.1 Architecture

Each input port maintains a single First-In-First-Out (FIFO) queue. Only the packet at the absolute head of the queue is eligible for switching in any given time slot. If the head packet's destination output is occupied by another input's packet, the head is blocked — and every packet behind it is also blocked, even if those packets could be served by idle outputs.

## 3.2 Head-of-Line (HoL) Blocking Explained

HoL blocking is the fundamental bottleneck of FIFO queuing in crossbar switches. It occurs when:

- The head packet of input Iₓ wants output Oy.

- Output Oy is already claimed by a lower-numbered input (tie-break rule).

- All packets behind the blocked head — even those destined for completely idle outputs — are forced to wait.

This wastes available output bandwidth and artificially inflates service time.

## 3.3 Step-by-Step Simulation Log

| Slot t | Packets Sent | HoL Blocking Observed | End Backlog |
| --- | --- | --- | --- |
| 0 | p1 (I₀→O₀) | p3 blocked (I₁→O₀ lost to I₀); p4 (I₁→O₂) stuck behind p3 despite O₂ idle | 4 |
| 1 | p2 (I₀→O₁), p3 (I₁→O₀) | p5 blocked (I₂→O₀ lost to I₁) | 4 |
| 2 | p6 (I₀→O₂), p5 (I₂→O₀) | p4 blocked (I₁→O₂ lost to I₀); p8 (I₁→O₁) stuck despite O₁ idle | 4 |
| 3 | p10 (I₀→O₁), p4 (I₁→O₂) | p8 blocked (I₁→O₁ lost to I₀); p11 stuck; p7 blocked (I₂→O₁ lost to I₀) | 5 |
| 4 | p13 (I₀→O₀), p8 (I₁→O₁) | p11 blocked; p14 stuck (O₂ idle); p7 blocked; p9 stuck (O₂ idle) | 6 |
| 5 | p16 (I₀→O₂), p11 (I₁→O₀), p7 (I₂→O₁) | p14 blocked (O₂ lost to I₀); p9 blocked | 6 |
| 6 | p14 (I₁→O₂) | p9 blocked (I₂→O₂ lost to I₁); p12 stuck (O₁ idle) | 5 |
| 7 | p17 (I₁→O₁), p9 (I₂→O₂) | p12 blocked; p18 stuck (O₀ idle) | 3 |
| 8 | p12 (I₂→O₁) | — | 2 |
| 9 | p15 (I₂→O₂) | — | 1 |
| 10 | p18 (I₂→O₀) | — | 0 |

✅  FIFO Total Service Time = 11 time slots

# 4. Theory — Virtual Output Queuing (VOQ)

## 4.1 Why VOQ Eliminates HoL Blocking

Instead of one shared FIFO, each input port maintains N separate queues — one per destination output. For a 3×3 switch, each input has 3 VOQs (e.g., I₀ maintains VOQ[I₀→O₀], VOQ[I₀→O₁], VOQ[I₀→O₂]).

Because each VOQ holds only packets for a single destination, a blocked packet in VOQ[Iₓ→Oy] never prevents packets in VOQ[Iₓ→Oz] from being served. HoL blocking is completely eliminated by design.

The scheduling problem then reduces to finding a valid bipartite matching at each time slot: a set of (input, output) pairs where no input and no output appears more than once. This is equivalent to finding an independent set in a bipartite graph.

## 4.2 Optimal Matching (Exhaustive Search)

At each time slot, all valid matchings (including partial ones) are enumerated. The matching that sends the maximum number of packets is selected. Among ties, the lexicographically smallest matching is chosen for determinism. This guarantees the theoretical minimum service time achievable by any VOQ-based scheduler.

# 5. Part 2 — VOQ Optimal (Exhaustive Search)

## 5.1 Step-by-Step Simulation Log

| Slot t | Matching (3 packets/slot) | End Backlog |
| --- | --- | --- |
| 0 | p2 (I₀→O₁), p4 (I₁→O₂), p5 (I₂→O₀) | 2 |
| 1 | p6 (I₀→O₂), p3 (I₁→O₀), p7 (I₂→O₁) | 1 |
| 2 | p1 (I₀→O₀), p8 (I₁→O₁), p9 (I₂→O₂) | 0 |
| 3 | p10 (I₀→O₁), p11 (I₁→O₀) [only 2 available] | 1 |
| 4 | p13 (I₀→O₀), p14 (I₁→O₂), p12 (I₂→O₁) | 1 |
| 5 | p16 (I₀→O₂), p17 (I₁→O₁), p18 (I₂→O₀) | 1 |
| 6 | p15 (I₂→O₂) [last packet] | 0 |

✅  VOQ-Optimal Total Service Time = 7 time slots

The optimal scheduler achieves 3 packets/slot for most slots. The only exception is t=3, where only 2 valid (non-conflicting) pairs exist among the remaining queued packets (p10→O₁ and p11→O₀ conflict on the same output with any third pair).

# 6. Theory — iSLIP Scheduling Algorithm

While exhaustive search finds the optimal matching, it is computationally infeasible in real hardware at line rate. iSLIP is a practical iterative algorithm that approximates maximum matching using round-robin arbitration, achieving excellent throughput with O(N) hardware complexity.

## 6.1 Architecture

- 3 Grant Arbiters — one per output port, each with a round-robin pointer.

- 3 Accept Arbiters — one per input port, each with a round-robin pointer.

- All pointers initialise to 0 and advance only when a grant is accepted.

## 6.2 Algorithm — One Iteration

- REQUEST: Each unmatched input requests all outputs for which it has packets in its VOQ.

- GRANT: Each unmatched output scans requesting inputs starting from its grant pointer (round-robin). It grants to the first requesting input found.

- ACCEPT: Each unmatched input scans the grants it received starting from its accept pointer. It accepts the first grant found and both pointers advance past the accepted pair.

Multiple iterations per slot allow inputs/outputs left unmatched in iteration 1 to be matched in subsequent iterations, improving the matching size towards the maximum.

## 6.3 Crucial Pointer Rule

Pointers advance ONLY when a grant is accepted — not on every grant. This desynchronises the arbiters over time, preventing the same input from always winning, and ensures long-term fairness across all ports.

# 7. Part 3 — iSLIP (3 Iterations per Slot)

## 7.1 Step-by-Step Simulation Log

| Slot t | Packets Sent | iSLIP Matching Detail | End Backlog |
| --- | --- | --- | --- |
| 0 | p1 (I₀→O₀), p4 (I₁→O₂) | Iter1: O0→I0, O1→I0, O2→I1; I0 accepts O0, I1 accepts O2. Iter2: O0→I2 (not taken — I2 has no O0 packet in remaining). 2 matched. | 3 |
| 1 | p2 (I₀→O₁), p3 (I₁→O₀) | Iter1: O0→I1, O1→I0, O2→I0; I0 accepts O1 (ptr=1), I1 accepts O0. Iter2: finds no unmatched pairs. | 3 |
| 2 | p8 (I₁→O₁), p5 (I₂→O₀), p6 (I₀→O₂) | Iter1: O0→I2, O1→I1, O2→I2; I1 acc O1, I2 acc O0. Iter2: O2→I0; I0 acc O2. Full 3-match. | 2 |
| 3 | p11 (I₁→O₀), p7 (I₂→O₁) | Iter1: O0→I1, O1→I2; I1 acc O0, I2 acc O1. Only 2 unmatched inputs. | 3 |
| 4 | p13 (I₀→O₀), p14 (I₁→O₂), p12 (I₂→O₁) | Iter1: O0→I0, O1→I0, O2→I1; I0 acc O0, I1 acc O2. Iter2: O1→I2; I2 acc O1. Full 3-match. | 3 |
| 5 | p10 (I₀→O₁), p9 (I₂→O₂) | Iter1: O0→I2, O1→I1, O2→I2; I1 acc O1, I2 acc O2. Iter2: O0→I0 (no packet). 2 matched. | 4 |
| 6 | p16 (I₀→O₂), p18 (I₂→O₀) | Iter1: O0→I2, O2→I0; I0 acc O2, I2 acc O0. 2 matched. | 1 (p15) |
| 7 | p15 (I₂→O₂) | Iter1: O2→I2; I2 acc O2. Last packet. | 0 |

✅  iSLIP Total Service Time = 8 time slots

# 8. Data Visualisation & Analysis

## 8.1 Graphs

The two graphs below compare total service time and backlog drainage across all three algorithms:

## 8.2 Backlog Over Time — Comparison Table

| Time Slot t | FIFO Backlog | VOQ-Optimal Backlog | iSLIP Backlog |
| --- | --- | --- | --- |
| 0 | 4 | 2 | 3 |
| 1 | 4 | 1 | 3 |
| 2 | 4 | 0 | 2 |
| 3 | 5 | 1 | 3 |
| 4 | 6 | 1 | 3 |
| 5 | 6 | 1 | 4 |
| 6 | 5 | 0 | 1 |
| 7 | 3 | — | 0 |
| 8 | 2 | — | — |
| 9 | 1 | — | — |
| 10 | 0 | — | — |

## 8.3 Analysis — Why FIFO Performs Worst

The FIFO line's backlog actually increases from t=0 to t=5 (peaking at 9 packets before sending), reaching a maximum end-of-slot backlog of 6 at slots t=4 and t=5. This growth happens because new packets arrive faster than FIFO can drain them, due to HoL blocking wasting output bandwidth. Specifically:

- p4 (I₁→O₂) is delayed 3 extra slots (t=0 to t=3) because it is stuck behind p3 which repeatedly loses the O₀ contest.

- p7 (I₂→O₁) is delayed 4 extra slots due to the same cascade effect.

- p9, p12, p18 form a long chain of secondary blocks, forcing single-packet slots at t=8,9,10.

VOQ eliminates this entirely — it achieves 3 packets/slot in 5 out of 7 slots, draining the switch in the theoretical minimum of 7 slots. iSLIP comes within 1 slot of optimal (8 vs 7), demonstrating its effectiveness as a practical hardware scheduler.

# 9. Final Results Comparison

| Algorithm | Service Time | Peak Backlog | HoL Blocking? |
| --- | --- | --- | --- |
| FIFO | 11 slots | 6 (end-of-slot) | Yes — severe (multiple instances) |
| VOQ-Optimal | 7 slots | 2 | None — eliminated by VOQ |
| iSLIP (3 iters) | 8 slots | 4 | None — eliminated by VOQ |

# 10. Conclusion

This simulation clearly demonstrates the impact of queue architecture and scheduling algorithm on switch performance:

- FIFO is the simplest to implement but suffers from severe Head-of-Line blocking. Output bandwidth is wasted whenever a blocked head packet prevents behind-it packets from using idle outputs. Service time of 11 slots is 57% worse than optimal.

- VOQ-Optimal completely eliminates HoL blocking and achieves the theoretical minimum service time of 7 slots by always finding the maximum bipartite matching. However, exhaustive search is impractical in real hardware.

- iSLIP with 3 iterations achieves 8 slots — only 1 slot worse than optimal — using a simple, hardware-friendly round-robin arbitration scheme. The pointer desynchronisation mechanism ensures fairness across ports over time.

For real network hardware, iSLIP represents the right balance: near-optimal throughput with practical O(N) complexity, making it widely used in high-speed crossbar switches.